Lone Star Logistics

Standard Operating Procedure (S.O.P.)

Rebuilding the 3-Site Network from Scratch in Cisco Packet Tracer

Prepared by: Kelvin Shepherd

# Contents

1. Purpose & Scope

2. Required Hardware & Addressing Plan

3. Procedure — Build the Topology

4. Procedure — Interface Addressing

5. Procedure — Static Routing

6. Procedure — OSPF Dynamic Routing

7. Procedure — DHCP Server & Relay

8. Procedure — Verification & Testing

9. Common Pitfalls & Troubleshooting Tips

10. Packet Tracer CLI Quirks Reference

11. Abbreviation & Terminology Glossary

# 1. Purpose & Scope

This document is a step-by-step, repeatable procedure for building the Lone Star Logistics 3-site network in Cisco Packet Tracer from an empty canvas — hardware placement, cabling, IPv4 addressing, static routing, OSPF, and DHCP — along with the specific mistakes made the first time through this project and how to avoid them. Follow the sections in order; each depends on the one before it.

Scope: Austin Headquarters (hub), Round Rock Sales Office (spoke), San Marcos Warehouse (spoke).

# 2. Required Hardware & Addressing Plan

## 2.1 — Devices per site

| Site | Router | Distribution Switch | Access Switch | End Devices |
| --- | --- | --- | --- | --- |
| Austin HQ | 1x Cisco 2911 (ATX-R) | 1x 2960-24TT (ATX-CSW) | 1x 2960-24TT (ATX-ASW) | 4x PC, 1x Server (inventory DB), 1x Server (DHCP), 1x Laptop (tech console) |
| Round Rock Sales | 1x Cisco 2911 (RR-R) | 1x 2960-24TT (RR-CSW) | 1x 2960-24TT (RR-ASW) | 2x PC |
| San Marcos Warehouse | 1x Cisco 2911 (SM-R) | 1x 2960-24TT (SM-CSW) | 1x 2960-24TT (SM-ASW) | 2x PC |

WATCH OUT: Use the 2960-24TT switch model at every layer, not the generic “Switch-PT.” Switch-PT ships with zero built-in copper ports until you manually add a network module (Physical tab → power off → drag in a module → power on). Building with 2960-24TT from the start avoids a rebuild later.

## 2.2 — IPv4 addressing plan

| Network | Purpose | Router Interface(s) |
| --- | --- | --- |
| 192.168.10.0/24 | Austin HQ LAN (staff, servers) | ATX-R Gig0/0 = .1 |
| 192.168.20.0/24 | Round Rock Sales LAN | RR-R Gig0/1 = .1 |
| 192.168.30.0/24 | San Marcos Warehouse LAN | SM-R Gig0/1 = .1 |
| 192.168.12.0/24 | WAN transit: Austin ↔ Round Rock | ATX-R Gig0/1 = .1, RR-R Gig0/0 = .2 |
| 192.168.13.0/24 | WAN transit: Austin ↔ San Marcos | ATX-R Gig0/2 = .1, SM-R Gig0/0 = .2 |

| Device | Static IP | Gateway |
| --- | --- | --- |
| DHCP Server (Server0) | 192.168.10.3 | 192.168.10.1 |
| ATX-Server / inventory DB (Server1) | 192.168.10.5 | 192.168.10.1 |
| Test Laptop (tech console) | n/a (console cable) or 192.168.10.x if also given an Ethernet drop | 192.168.10.1 |

# 3. Procedure — Build the Topology

- Place three 2911 routers on the canvas, one per site: ATX-R, RR-R, SM-R.

- Place two 2960-24TT switches per site (a distribution/core switch and an access switch), e.g. ATX-CSW + ATX-ASW, RR-CSW + RR-ASW, SM-CSW + SM-ASW.

- Place end devices: 4 PCs + 2 servers + 1 laptop at HQ; 2 PCs at Sales; 2 PCs at Warehouse.

- Cable each router's LAN interface to that site's distribution switch, and the distribution switch down to the access switch, and the access switch out to the PCs — mirror this pattern identically at all three sites so the topology stays easy to reason about.

- Cable the two WAN links: ATX-R ↔ RR-R, and ATX-R ↔ SM-R, using a second and third router-facing interface on ATX-R (it needs three usable interfaces total: one LAN, two WAN).

- Connect the DHCP server and the inventory-database server (ATX-Server) to the HQ distribution switch (ATX-CSW), not the access switch — this keeps infrastructure/servers logically separate from end-user PCs.

- Connect the Test Laptop to a router via console cable for CLI access. If it should also act as a test host on the network, give it a second, separate Ethernet connection to a switch — a console cable alone will never get it an IP address.

WATCH OUT: Before assigning any IP address or running any command, hover over every cable in the topology and confirm exactly which physical interface is on each end (e.g. “Router1 GigabitEthernet0/1 <-----> Router0 GigabitEthernet0/0”). Packet Tracer will happily let you cable the wrong port, and every later step depends on knowing the true mapping. Run show ip interface brief on each router first to see what interfaces actually exist before assuming names like Gig0/0/Gig0/1/Gig0/2.

# 4. Procedure — Interface Addressing

Repeat for ATX-R, RR-R, and SM-R using the addressing plan in Section 2.2. Example shown for ATX-R:

enable

configure terminal

hostname ATX-R

interface GigabitEthernet0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/1

ip address 192.168.12.1 255.255.255.0

no shutdown

exit

interface GigabitEthernet0/2

ip address 192.168.13.1 255.255.255.0

no shutdown

exit

end

copy running-config startup-config

Verify with:

show ip interface brief

Every interface you configured should show its correct IP and up / up. A WAN link will show down / down on one end until the far router's matching interface is also configured and no shutdown — this is expected and resolves itself once both ends are done.

TIP: If a WAN interface still shows down / down after both ends are configured and no shutdown has been run, don't assume the config is wrong — check the physical cable first. Delete and redraw a fresh Copper Straight-Through between the exact two ports; a stale/duplicated cable from earlier topology edits is a common, config-invisible cause.

# 5. Procedure — Static Routing

Austin is the hub; Round Rock and San Marcos are single-homed spokes. Static routing needs: (a) HQ reaching both branch LANs, (b) each branch reaching HQ, and (c) each branch reaching the OTHER branch's LAN through HQ. Do not stop at (a) and (b) — (c) is easy to miss and is exactly the failure the trouble-ticket scenario tests for.

## 5.1 — ATX-R

ip route 192.168.20.0 255.255.255.0 192.168.12.2

ip route 192.168.30.0 255.255.255.0 192.168.13.2

## 5.2 — RR-R

ip route 192.168.10.0 255.255.255.0 192.168.12.1

ip route 192.168.30.0 255.255.255.0 192.168.12.1

ip route 192.168.13.0 255.255.255.0 192.168.12.1

WATCH OUT: The third line above — a route to the OTHER branch's WAN transit subnet (192.168.13.0/24) — is the one most people forget. Without it, a ping or traceroute run directly FROM a router (sourced from its own WAN interface) to the far branch will fail, even though PC-to-PC traffic across the same path may still work. Add this route on both spokes from the start; don't wait to discover it's missing.

## 5.3 — SM-R

ip route 192.168.10.0 255.255.255.0 192.168.13.1

ip route 192.168.20.0 255.255.255.0 192.168.13.1

ip route 192.168.12.0 255.255.255.0 192.168.13.1

## 5.4 — Save and verify on all three routers

end

copy running-config startup-config

show ip route

## 5.5 — Verification ping/traceroute checklist

- ATX-R → 192.168.20.1 and → 192.168.30.1 (both should succeed)

- RR-R → 192.168.30.1 (Sales reaching Warehouse through HQ)

- SM-R → 192.168.20.1 (Warehouse reaching Sales through HQ)

- traceroute from RR-R to 192.168.30.1 should show exactly 2 hops: ATX-R, then SM-R

TIP: A single failed/timed-out ping (shown as one leading '.' before a run of '!') on the FIRST attempt to a brand-new destination is normal — it's just the ARP-resolution delay for a fresh entry. Re-run the same ping once before assuming something is broken.

# 6. Procedure — OSPF Dynamic Routing (Optional / Bonus)

OSPF can run alongside the static routes above without conflict — static routes (administrative distance 1) will still be preferred over OSPF routes (administrative distance 110) for the same destination, so adding OSPF is safe and reversible at any time.

! ATX-R:

router ospf 1

network 192.168.10.0 0.0.0.255 area 0

network 192.168.12.0 0.0.0.255 area 0

network 192.168.13.0 0.0.0.255 area 0

! RR-R:

router ospf 1

network 192.168.20.0 0.0.0.255 area 0

network 192.168.12.0 0.0.0.255 area 0

! SM-R:

router ospf 1

network 192.168.30.0 0.0.0.255 area 0

network 192.168.13.0 0.0.0.255 area 0

WATCH OUT: router ospf requires a process ID number (e.g. router ospf 1) — typing router ospf alone returns “% Incomplete command.” And the network ... area 0 lines MUST be typed while the prompt reads (config-router)#, not (config)# — if you see “% Invalid input detected,” you likely dropped out of router configuration mode without noticing.

Verify neighbor adjacencies (allow 30–60 seconds, or use Fast Forward Time, for the DR/BDR election to finish — an interface may briefly show State WAITING with Neighbor Count 0 before settling):

show ip ospf neighbor

show ip ospf interface <interface-name>

Expected result: ATX-R shows two neighbors (Round Rock and San Marcos); each spoke shows exactly one neighbor (Austin).

TIP: show ip route ospf will legitimately show NOTHING if static routes to the same networks already exist — this is not a failure. Confirm OSPF is actually working with show ip ospf neighbor (adjacency state FULL) and show ip ospf database (link-state entries from all routers) instead.

# 7. Procedure — DHCP Server & Relay

## 7.1 — Give the DHCP server (and any other HQ servers) a static IP first

Server0 (DHCP): Desktop tab → IP Configuration → Static → 192.168.10.3 / 255.255.255.0 / gateway 192.168.10.1.

Server1 (ATX-Server / inventory DB): same tab → Static → 192.168.10.5 / 255.255.255.0 / gateway 192.168.10.1.

## 7.2 — Create three DHCP pools on the server

Server0 → Services tab → DHCP → confirm the service is On → add each pool below, clicking Save after each one:

| Pool Name | Default Gateway | Start IP Address | Subnet Mask | Max Users |
| --- | --- | --- | --- | --- |
| HQ-POOL | 192.168.10.1 | 192.168.10.10 | 255.255.255.0 | 50 |
| SALES-POOL | 192.168.20.1 | 192.168.20.10 | 255.255.255.0 | 50 |
| WAREHOUSE-POOL | 192.168.30.1 | 192.168.30.10 | 255.255.255.0 | 50 |

WATCH OUT: Packet Tracer auto-creates a default pool called “serverPool” the first time DHCP is enabled, with gateway 0.0.0.0 and start address 192.168.10.0 (a network address, not a valid host). Its subnet overlaps HQ-POOL. If a PC on the HQ LAN comes back with a blank/0.0.0.0 default gateway instead of 192.168.10.1, THIS is almost always the cause — select serverPool in the pool table and click Remove. If Remove is unresponsive, toggle the DHCP service Off, remove it, then toggle back On.

## 7.3 — Enable DHCP relay on both spoke routers

DHCP client broadcasts never cross a router hop on their own, so each spoke's LAN-facing interface must relay them to the server at HQ:

! On RR-R:

interface GigabitEthernet0/1

ip helper-address 192.168.10.3

! On SM-R:

interface GigabitEthernet0/1

ip helper-address 192.168.10.3

WATCH OUT: Double-check ip helper-address is applied to the LAN-facing interface (toward the access switch), not the WAN-facing interface toward the other router. Placing it on the wrong interface, or on ATX-R at all (its LAN is already on the same subnet as the DHCP server — no relay needed there), does no harm but also does nothing useful.

## 7.4 — Switch every PC to DHCP

On each PC: Desktop tab → IP Configuration → click DHCP. If the fields stay blank, toggle Static then back to DHCP to force a fresh request, or use Fast Forward Time if it sits on “Requesting IP Address” for more than ~15–20 seconds.

# 8. Procedure — Verification & Testing

- show ip interface brief on all three routers — every configured interface shows up / up.

- show ip route on all three routers — confirm the static routes from Section 5 are present with the S code.

- Ping every branch LAN gateway from ATX-R (192.168.20.1, 192.168.30.1) — both succeed.

- Ping the far branch from each spoke router (RR-R → 192.168.30.1, SM-R → 192.168.20.1) — both succeed; use traceroute to confirm a clean 2-hop path if either fails.

- Set every PC to DHCP and confirm each one lands in its own site's subnet with the correct gateway (Section 7.2 table).

- From an actual PC's Command Prompt (not the router CLI), ping across sites: Sales PC → Warehouse PC, Warehouse PC → HQ PC, HQ PC → Sales PC, and any PC → both HQ servers (192.168.10.3, 192.168.10.5).

- If OSPF was configured, run show ip ospf neighbor on all three routers and confirm HQ shows two neighbors while each spoke shows one.

A 75–80% ping success rate on the FIRST test to a brand-new destination (one dropped packet, rest successful) is normal ARP behavior, not a fault — re-run once if you want a clean 100% result for a screenshot.

# 9. Common Pitfalls & Troubleshooting Tips

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| PC's DHCP request gets gateway 0.0.0.0 | Overlapping default “serverPool” on the DHCP server | Remove serverPool (Section 7.2) |
| A WAN interface stays down/down despite correct config | Stale or mis-plugged cable from earlier topology edits | Delete and redraw the cable between the exact two ports |
| Router-sourced ping to the far branch fails, but PC-to-PC across the same path works | Missing static route to the OTHER branch's WAN transit subnet | Add the third static route on each spoke (Section 5.2/5.3) |
| “% Invalid input detected” on a command you know is correct | Command run at the wrong prompt level, or the router's own prompt text was pasted along with the command | Type only the bare command — never include the displayed prompt (e.g. Router(config)#) as part of what you paste |
| OSPF interface stuck in State WAITING, Neighbor Count 0 | DR/BDR election still in progress (needs the Dead-timer window to elapse) | Wait ~40 seconds or use Fast Forward Time, then re-check |
| show ip route ospf returns nothing | Static routes (AD 1) to the same networks outrank OSPF (AD 110) — expected, not a bug | Verify with show ip ospf neighbor / show ip ospf database instead |
| A generic switch has no ports to cable into | Switch-PT ships with zero built-in ports | Add a network module (Physical tab) or replace with a 2960-24TT |
| Device shows “Device is booting” and CLI is unresponsive | Normal boot delay in the simulation | Wait 10–15 seconds, then retry |

# 10. Packet Tracer CLI Quirks Reference

- Type each command on its own line — never paste a router's displayed prompt (e.g. ATX-R(config)#) together with the command that follows it. Packet Tracer reads the whole pasted string as one command and rejects it with “% Invalid input detected.”

- router ospf needs a process ID: router ospf 1, not router ospf alone.

- network ... area 0 statements only work inside router configuration mode — confirm the prompt reads (config-router)# before typing them.

- show run and show run interface <name> are sometimes rejected; use the unabbreviated show running-config and scroll through the --More-- prompts (press spacebar) if a shorthand form errors out.

- Extended ping (typing ping with nothing else, then answering each prompt in turn) is the only way to test a specific source interface/address — ping <ip> source <ip> is not valid inline syntax on these images.

- Ctrl+Shift+6 aborts a running ping/traceroute early if needed, but letting it finish on its own is usually simpler.

- arp -d on a PC's Command Prompt clears its ARP cache — useful for forcing a fresh ARP Request to appear in Simulation mode instead of reusing a cached MAC address.

- Reset Simulation (Simulation panel) clears stale event-list state after a cable/topology edit — worth doing before re-testing if a link was just changed.

# 11. Abbreviation & Terminology Glossary

| Abbreviation | Meaning |
| --- | --- |
| ARP | Address Resolution Protocol — resolves an IP address to a MAC address |
| ASW | Access Switch — switch layer end-user devices plug into |
| CSW | Core/Distribution Switch — switch layer between the router and the access switch |
| DR / BDR | Designated Router / Backup Designated Router — OSPF's elected roles on a broadcast segment |
| DHCP | Dynamic Host Configuration Protocol — automatically assigns IP addressing to clients |
| FA / Fa | FastEthernet interface |
| GE / Gig | GigabitEthernet interface |
| HQ | Headquarters (Austin site in this project) |
| ICMP | Internet Control Message Protocol — carries ping (Echo Request/Reply) |
| LAN | Local Area Network |
| LSA | Link-State Advertisement — the routing information OSPF routers exchange |
| MAC | Media Access Control address — Layer 2 hardware address |
| OSPF | Open Shortest Path First — a link-state dynamic routing protocol |
| PDU | Protocol Data Unit — the packet/frame inspected in Packet Tracer's Simulation mode |
| RR | Round Rock (Sales Office site in this project) |
| SM | San Marcos (Warehouse site in this project) |
| TTL | Time to Live — IP header field decremented by 1 at every router hop |
| WAN | Wide Area Network — the router-to-router links between sites |

— End of S.O.P. Manual —